The eth_axis_tx module is a  implementation of an Ethernet frame transmitter over an AXI4-Stream interface. It takes Ethernet header fields (destination MAC, source MAC, Ethertype) and AXI-streamed payload data as input, combines them, and outputs a full Ethernet frame via the AXI stream interface. Here's a detailed explanation of its functionality:

🔧 Module Parameters

- DATA_WIDTH: Width of the AXI stream data interface.

- KEEP_ENABLE: If 1, the tkeep signal is used to indicate valid bytes.

- KEEP_WIDTH: Width of tkeep = DATA_WIDTH / 8.

📥 Inputs

- s_eth_hdr_valid: Indicates header is valid.

- s_eth_hdr_ready: Handshake signal; high when ready to accept a new header.

- s_eth_dest_mac, s_eth_src_mac, s_eth_type: Ethernet header fields (MACs + Ethertype).

- AXI-stream signals:

  - s_eth_payload_axis_tdata: Payload data.

  - s_eth_payload_axis_tkeep: Byte qualifiers.

  - s_eth_payload_axis_tvalid, tready, tlast, tuser: Standard AXI stream signaling.

- clk, rst: Clock and reset.

📤 Outputs

- AXI-stream output: m_axis_tdata, tkeep, tvalid, tlast, tuser.

- busy: Indicates the module is actively processing data.

🧠 Internal Functionality Overview

📦 1. Header Handling

- When s_eth_hdr_valid and s_eth_hdr_ready are both high:

  - The header fields are stored in internal registers.

  - A state machine begins transmitting the 14-byte header (6B dest + 6B src + 2B type).

  - Header transmission takes CYCLE_COUNT cycles based on DATA_WIDTH.

🚚 2. Payload Handling

- Payload data is forwarded after header transmission is complete.

- Handles data alignment if HDR_SIZE isn't a multiple of data bus width (OFFSET logic).

- Manages a “save” and “shift” mechanism to align and merge leftover payload bytes with new ones across cycles.

🔁 3. State Machine

Two main states:

- send_eth_header_reg: Asserts during header transmission.

- send_eth_payload_reg: Asserts during payload transmission.
Transitions are managed by internal pointers (ptr_reg) and offset conditions.

📊 4. AXI Stream Output Interface

- Internally buffers output data using a two-level staging:

  - Registers for direct output (m_axis_tdata_reg, etc.).

  - Temporary registers (temp_m_axis_tdata_reg, etc.) to hold intermediate data when downstream is not ready.

- Implements a logic block to manage ready/valid handshake between internal logic and AXI output.

🧾 Key Logic Highlights

✅ AXI Header Construction:

Header bytes are assembled and streamed based on current ptr_reg:

verilog

CopyEdit

if (ptr_reg == offset/BYTE_LANES) begin

m_axis_tdata_int[...] = field;

m_axis_tkeep_int[...] = 1'b1;

end

📉 tkeep and Offset Logic:

When the header ends mid-beat (e.g., on 3rd byte of 8-bit lane), the logic:

- Saves the remaining payload.

- Merges new payload with saved payload and aligns it.

- Adjusts tkeep accordingly to indicate valid byte lanes.

🏁 End-of-Frame (EOP) Handling:

- Sets tlast when the final beat of the payload is detected.

- Uses s_eth_payload_axis_tlast and tkeep bits to determine whether the current beat is final or needs an extra cycle.

🧮 Example Use Case (DATA_WIDTH = 64):

- KEEP_WIDTH = 8 (1 byte per bit in tkeep)

- Ethernet header (14B) fits into 2 full 8-byte cycles.

- If payload is streamed in next cycles, AXI stream will first contain:

  - 8B header

  - 6B header + 2B payload

  - Next payload bytes

🔄 Reset Behavior

On reset (rst high), the module:

- Clears internal state and output registers.

- Resets header pointer and control FSMs.

Bock diagram for eth_axi_tx