🔹 1. eth_mac_10g.v – Top-Level 10G Ethernet MAC Module

✅ Purpose:

This is the top-level wrapper module for the 10G Ethernet MAC. It integrates:

- Transmit and receive logic

- Flow control (pause frame support)

- MAC control (optional)

- XGMII interface on one side and AXI-Stream on the other

📦 Submodules Likely Included:

- axis_xgmii_rx_64 or axis_xgmii_rx_32 (based on data width)

- axis_xgmii_tx_64 or axis_xgmii_tx_32

- mac_pause_ctrl_rx

- mac_pause_ctrl_tx

- mac_ctrl_rx (optional MAC control logic)

- mac_ctrl_tx (optional MAC control logic)

⚙️ Interfaces:

- XGMII TX/RX: 64-bit or 32-bit wide bus, based on mode

- AXI-Stream TX/RX: For user logic to transmit/receive Ethernet frames

- Flow Control: Optional pause frame support using mac_pause_ctrl_* modules

🔹 2. axis_xgmii_rx_64.v and axis_xgmii_rx_32.v – XGMII Receiver to AXI-Stream

✅ Purpose:

- Converts XGMII (64-bit or 32-bit) receive data into AXI-Stream Ethernet frames.

- Detects preambles, Start-of-Packet (SOP), End-of-Packet (EOP)

- Removes inter-frame gaps (IFG)

- Reports errors (e.g., bad FCS, early termination)

🔁 Differences:

- _rx_64.v: Used in 64-bit XGMII mode (common for 10G)

- _rx_32.v: Used in 32-bit mode (e.g., for simulation or resource-constrained designs)

⚙️ Key Operations:

- Watches for SFD (Start Frame Delimiter)

- Extracts destination/source MAC and ethertype

- Validates frame structure and optionally CRC

- Sends data via AXI-Stream output ports

🔹 3. axis_xgmii_tx_64.v and axis_xgmii_tx_32.v – AXI-Stream to XGMII Transmitter

✅ Purpose:

- Takes AXI-Stream Ethernet frames and sends them over XGMII

- Adds preamble, Start Frame Delimiter (SFD), pads frame if needed

- Inserts IFG (inter-frame gap)

- Calculates and appends CRC (FCS)

🔁 Differences:

- _tx_64.v: Works with 64-bit XGMII (most common)

- _tx_32.v: Works with 32-bit XGMII interface

⚙️ Key Operations:

- Serializes AXI frame into XGMII data/control characters

- Inserts IDLE characters when not transmitting

- Ensures compliance with IEEE 802.3

🔹 4. mac_ctrl_rx.v – MAC Control RX

✅ Purpose:

- Handles reception of special MAC control frames (e.g., pause frames)

- Detects opcode = 0x0001 which signifies a pause frame

- Extracts pause time value

- Passes control info to the pause control logic

🔹 5. mac_ctrl_tx.v – MAC Control TX

✅ Purpose:

- Builds and sends MAC control frames like pause frames

- Triggered by internal flow control logic or external logic

- Encodes frame with:

  - DA: 01-80-C2-00-00-01 (reserved multicast for pause)

  - Opcode: 0x0001

  - Pause time: 16-bit value

🔹 6. mac_pause_ctrl_rx.v – Pause Frame Receiver Control

✅ Purpose:

- Handles the logic once a pause frame is received

- Interprets pause time and applies it to internal pause state

- Coordinates pause signal assertion/deassertion based on timers

🔹 7. mac_pause_ctrl_tx.v – Pause Frame Transmitter Control

✅ Purpose:

- Generates pause request logic

- Handles pause frame transmit timing and interval enforcement

- Works in conjunction with mac_ctrl_tx.v

| File | Function |
| --- | --- |
| axis_xgmii_rx_32.v / axis_xgmii_rx_64.v | Converts XGMII RX interface to AXI4-Stream — decodes received Ethernet frames. Supports 32- or 64-bit XGMII data bus width. |
| axis_xgmii_tx_32.v / axis_xgmii_tx_64.v | Converts AXI4-Stream TX data to XGMII TX interface — adds preamble, CRC, and encodes Ethernet frames. |
| eth_mac_10g.v | Top-level 10G Ethernet MAC wrapper — connects RX/TX datapaths, pause, and control logic. |
| mac_ctrl_rx.v | Handles incoming MAC control frames (like pause frames) from the network. |
| mac_ctrl_tx.v | Sends MAC control frames (pause, etc.) to the network. |
| mac_pause_ctrl_rx.v | Processes RX pause frames — extracts pause time and generates flow control. |
| mac_pause_ctrl_tx.v | Implements TX-side pause frame generation — manages pause request timing. |

Block Diagram – 10G Ethernet MAC TX & RX

Hierarchy Flow:

+----------------------+

|     eth_mac_10g      |

|----------------------|

|                      |

+--------v--------+   +---------v-------+

| axis_xgmii_tx_32|   | axis_xgmii_rx_32|

| axis_xgmii_tx_64|   | axis_xgmii_rx_64| <- Optional

+--------+--------+   +---------+-------+

|                      |

|                      |

+---v---+              +---v---+

| XGMII |              | XGMII |

|  PHY  |              |  PHY  |

+-------+              +-------+

Control Path:

+--------------------+

| MAC Pause/Control  |

| mac_ctrl_tx / rx   |

| mac_pause_ctrl_tx  |

| mac_pause_ctrl_rx  |

+--------------------+

TX Waveform Example (axis_xgmii_tx_64)

Explanation:

- First 8B: Preamble + SFD

- Next 2 cycles: Ethernet header (14B)

- Payload: Starts after header

- tlast: Indicates end of frame

RX Waveform Example (axis_xgmii_rx_64)